GLOBAL ANALYSIS OF STRUCURES IN FIRE

The Need, the State of the Art and Future Trends.

Notes for COST-C26 Datasheet

Dr M Gillie

Prescriptive based design requires understanding of global structural behaviour, furnace test results etc no longer good enough.  This means global modelling of structures is required:

# Why?

Ambient temperature analysis and design for static loads (including wind)

- Actions are mechanical loads

- These lead directly to stresses

- Stresses constant for the duration of the action

- Deflections must be small, or excessive stresses and/or serviceability failure result

- This allows for individual member analysis (generally true, overall stability etc needed)

- Note: redundant structures often use a degree of global analysis – portal frames etc.

Fire analysis and design

- Actions are temperatures (heat fluxes?)

- These lead to stresses varying over the duration of the action and…

- …thermal expansion resulting in large deflections at low stresses - not necessarily failure.

- Individual member analysis therefore not possible because forces transferred between members change during action and…

- …large deflections can not be represented well in an individual member analysis due to changing (and uncertain) support conditions.

- This all implies global modelling

Statement of when global modelling essential and when local modelling suitable.

# Important Features and Good Practice

  - Modelling of structures in fire a challenging task

  - Many nonlinear phenomena

  - At limits of what is possible with current software

  - Consideration must be given to all of the following points for a reliable analysis to result

  - Results can be counter-intuitive – distinguishing between errors and unexpected effects difficult.

## Software

- Choice between commercial software: Abaqus, Ansys, Oasys etc…

- …and research based codes Vulcan, Safir etc

- Commercial codes faster, better validation and verification (perhaps needed for checking authorities), can be used for modelling other problems.  But very expensive and blackboxes

- Research codes cheaper, dedicated to fire, code (or nature of code) easily accessible.  But less able to cope with very large problems

## Geometric Nonlinearity

- Vital.

- Large deflections play an important role in almost all structural fire problems.

- No analysis will be reliable without consideration of geometrically nonlinear effects.

## Static/Dynamic Modelling

- Until recently almost all modelling undertaken using static codes

- Approach sound as inertia effects small until failure

- However, local buckling often leads to numerical problems early in the analysis.

- This does not necessarily indicate failure because probably displacement controlled

- Dynamic modelling very powerful and does not suffer from stability problems (much!) and can model failure process as inertia effects included.

- Care needed as only conditionally stable.

- Wise to conduct both types of analysis and compare results.

## Material Modelling

### Steel

- Fairly well-behaved and well understood

- Stress, strain, thermal expansion data all widely available eg Eurocodes.

### Concrete

- Complex material behaviour not fully understood at high temps (or ambient temps?)

- Different behaviour in tension and compression must be modelled

- Representing crushing and cracking numerically difficult

- Smeared cracking and approximations may be possible if details of local behaviour will not affect global response…

- …mostly true of steel-concrete composite type structures.  Validity for other structural forms less clear

- Research area, much to be done

## Loads and Boundary Conditions

- Good estimate of temperatures and temperature gradients needed (huge topic in its own right)

- Boundary conditions should be used with care.

# Current possibilities

- Modelling of steel structures

- Modelling of composite structures e.g. Cardington

- Modelling of collapse e.g WTC modelling

- Design of composite structures e.g. Arup projects

- Should include a wider perspective here – Ian’s and Jean-Marc’s projects and others

# Future needs and current limitations?

- Modelling of local effects not currently included in models: connections, shear stud behaviour, effect of web openings need to be included without killing the model

- Concrete behaviour (need test data, agreement on constitutive models)

- Cooling (in hand)

- Travelling fires

# Benchmarking Problems

- Benchmarking problems would be useful if new codes are being used or for new practitioners to establish they are producing good models.