PRACTICAL  QUESTIONS  CONCERNING  DESIGN  AND/OR USE  OF  BOLTS  IN  RELATION  WITH  EC3

(written by M. Braham, 11.07.2001)

Following questions are addressed by Marc Braham (ASTRON, Luxembourg)

| Q058 | - What is the influence of gaps ( out of flatness of plates due to welding shrinkage for example) in connections submitted to bending in case of static or dynamic loading. |
| --- | --- |
| Q059 | - In case a connection is submitted to very low forces, is it still necessary to design it for a certain "reasonable" level of forces, as compared to the resistance of the beam/column for example. Differentiate between elastic and plastic design. |
| Q060 | - As it is quite difficult, even almost impossible, to pre-stress a bolt on site in case of modest construction (industrial buildings for example), wouldn’t it be interesting to specify a reduced pre-stress force for these cases? For example 50% of nominal pre-stress load. |
| Q061 | - Why to limit yield strength of anchor bolt to 300 N/mm2 in case of anchorage with hook (paragraph 5.2.6.12 in EC3 part 1.8). This should be removed from EC3. |
| Q062 | - How to extend EC3 rules to bolt-rows with 4 bolts instead of 2. |
| Q063 | - Is there any possibility to use pre-stressed bolts with not prepared friction surfaces (surfaces are painted, or rusty) |

Following questions are addressed by J. Stark (NL)

| Q064 | - Frames are usually modeled with a system of lines, joining centers of gravity of sections. In the figure below, if the connection is designed as pinned may the column be designed as a centrally loaded column? |
| --- | --- |
| Q065 | - Is it permitted to use a plastic distribution of internal forces for a partial strength beam-to-column connection if very thick end-plates are used? If not, is there any criteria for what thickness elastic design is required? |
| Q066 | - Is it necessary to take into account all stress concentrations for the fatigue verification of welds? |
| Q067 | - An end plated connection as shown in figure below is normally loaded by a moment and a shear force. How is the shear force distributed over the bolts? |
| Q068 | - What is the minimum thickness of welds in unstiffened beam-to-column connections? |
| Q069 | - In ENV1993 four classes A,B,C, D are introduced for the friction coefficient of contact surfaces in slip resistant connections. What is the use of these classes if the friction coefficient is determined with a test procedure? |
| Q070 | - In ENV1993, design rules are given for connections made with pins. What is the difference with a single bolted connection and what is the background for the formulae given for the pinned connections? |
| Q071 | - During erection a gap of 10 mm between an end-plate of a beam and the column-flange is discovered. Is it permitted to fill this gap with filler plates? |
| Q072 | - A connection with pre-loaded bolts is subjected to tension forces. How can tension forces on the bolts be determined, and what is the design criteria? |
| Q073 | - Is it permitted to use single sided fillet welds for the connection of the webs and the bottom flange of integrated beams for slim floor construction as shown in the figure? |
| Q074 | - What is the design value for the tensile resistance if M20 bolts loaded in tension are used in oversized holes 24 mm? |
|  | - In the figure, a yield line pattern is given for a bolt group in an unstiffened flange. Is such a pattern a realistic option in bolted end plate connection? |

Following questions are addressed by ASTRON (Luxembourg)

| Q075 | - Is it possible to transmit shear forces through long holes (slotted holes)? Is it necessary to use pre-stressed bolts for this? |
| --- | --- |
| Q076 | - In case of static loading, the resistance of a connection is calculated on the basis of the resistance of the bolt rows (including effect of the plate). There is thus no need to discuss about "what is the load in a bolt for a certain loading?". However, in case of fatigue, there is a need to know or define the , in order to be able to verify the resistance of the bolt for fatigue. |
| Q077 | - T stub columns behaviour is known and is included in the calculation of the resistance of a bolt row: this is however for static loading. In case of fatigue it is necessary to guess for a distribution of the load in order to define stresses and verify resistance. |
| Q078 | - Lever arm corresponding to prying forces are included in formulas for resistance of bolt rows. However, in case of fatigue the effect of prying forces should be known in the bolts in order to verify the bolts. How to cope with this? |
| q079 | - Backing plates are included in the calculation of resistance of bolt-rows in ENV. However, how to deal with these plates and their effect in case of fatigue? |
| Q080 | - In order to make sure that a connection is properly assembled, and has no undesirable flexibility due to bad erection, it is quite interesting to prescribe a certain tightening. However, real measuring of tightening is not easy, even difficult and expensive, and also this is not necessary. What is thus the best instruction for the job-site in usual industrial building erection? Use 50 % torque, use combined method, ...? Consider also that plates are not perfectly flat. |
| Q081 | - In case of welded beam-end-plate, there is quite often an unflatness of the plate due to weld shrinkage. What is thus the best tightening method in that case, when real pre-stressing is required? |

Questions addressed through E. Piraprez (CRIF, Belgium)

| Q082 | - Which is the best lubricant for bolts in stainless steel, giving a low and constant k coefficient? |
| --- | --- |
| Q083 | - At low temperature, what is the adequate lubricant to achieve a good tightening of the bolts? Until which (low) temperature can it be used? |
| Q084 | - At low temperature, a normal bolt tigtened at room temperature has a good behaviour until which (low) temperature? |
| Q085 | - How to design a beam-to-column connection when the normal force in the beam is high? |
| Q086 | - How to obtain resistance and design pre-load of bolts with a larger > 42 mm diameter? |
| Q087 | - What is the slip factor between steel and concrete? |