# COMMENTS ON ECCS STARTER DRAFT FOR JOINTS

| # Clause No | # Country | # Remarks and comments | # No. | # Class | # Proposed action |
| --- | --- | --- | --- | --- | --- |
| 2.1.2 | UK | This clause states in the heading that it is about minimum end distances yet 2.1.2(2) includes information on edge distances. The headings must be consistent with the information given in the clause. Where are the stress fluctuations? | 1 | E | Change the title of this clause to Minimum end and edge distances and combined clauses 2.1.2 and 2.1.3 into one clause. Define stress fluctuations. I.e. Where the connection is subject to impact, vibration or load reversal …. |
| 2.1.4 (2) | UK | What is the background to the maximum end and edge distance of 12t + 150mm? Is this for structural reasons or just aesthetics? If the latter please remove the clause. | 2 | P | Remove this clause. |
| 2.1.7 (2) | UK | Which values can be multiplied by 1.5? | 3 | E | Rewrite this clause ' If the members are not exposed to corrosive influences, the maximum spacing between the inner bolts, p1,I, should not exceed the lesser of 42t or 600mm provided that P1,0 in the outer row along each edge does not exceed the lesser of 21t or 300mm. |
| 2.2.1 (2) | ## UK | This clause seems to imply that ordinary bolts are bolts made from low carbon steel. Please define ordinary bolts. | 4 | E | Either include a reference to standard for bolts or omit the words '(manufactured from low carbon steel)'. |
| 2.2 | ## UK | Does the heading ‘Connections subjected to shear forces’ refer to the connection or the components within the connection? For example is the connection subject to shear force only or are the components in the connection subject to shear only? If the latter then the connection can be subject to a combination of applied load. (e.g. the fin plates shown in Fig. 2.9 are subject to combined shear and moment.). | 5 | AM | Suggest that the whole of Chapter 2 is re-structured using the following headings:- 2.1 Connections using bolts 2.1.1 Bolt spacing 2.1.2 Edge and end distances 2.1.3 Block shear - Non-preloaded bolts - Shear capacity - Bearing capacity - Bolts subject to tension 2.4. Preloaded bolts 6.5 Pin connections |
| 2.2.3 (5) | ## UK | Why is the shear strength of M12 and M14 bolts reduced by 0.85? | 6 | R | Reduction is based on tests./ECCS/TC10 |
| 2.2.3 (8) | ## UK | Where are the more accurate values for the design bearing resistance of a bolt? | 7 | E | Rewrite this clause. 'For bolts in standard clearance holes (see ??) the design bearing resistance, Fb, Rd may be obtained from Table 2.3.' |
| 2.2.6 | ## UK | Injection bolts are rarely used therefore remove the whole of this clause. | 8 | R | This form of bolt is use in bridge repair. |
| 2.2.8.2 (1) | ## UK | This is only one way of dealing with the problem of block shear. An alternative is to accept block shear as the governing mode of failure and reduce the resistance of the connection appropriately. Or a thicker web could be used. This is an example of where a principle would be more helpful than a limited application rule that restricts the freedom of the designer. | 9 | A | Include a new clause' Block shear failure through a group of bolt holes at a free edge, see figure 2.7, should be prevented by checking that the reaction does not exceed the effective block shear capacity.' |
| 2.2.8.4 | ## UK | Lug angles are rarely used. Remove this clause. | 10 | R | The UK asked for this to be included. |
| Figure 2.9 | ## UK | This figure should be headed examples as it gives examples of possible distributions of load between fasteners. Note that the figure shows a fin plate subject to combined shear and moment but the title of section 2.2 is ‘Connections subjected to shear forces’. | 11 | A | Use a different figure. (To be discussed with members of the ad-hoc group). |
| 2.2.10 = 3.3.1(12) of Part 1.8 | ## UK | Remove this clause – not necessary. | 12 | AM | The rule is needed. Add note: This rule need not be applied, if the joint is supporter against lateral deformations. |
| 2.2.12 (3) | ## UK | What are the additional fasteners for – to hold the packing in position? As worded the additional bolts need not be used to transmit the force through the packing but can be used to hold the packing in position. | 13 | E | Delete this clause. Firstly, as written it is ambiguous and secondly the designer can decided where to put any additional bolts. |
| 2.3 | ## UK | Again does the heading refer to connection subject to tension or to tension forces applied to the components of the connection? | 14 | A | See comment on clause 2.2 (comment no. 5). |
| Table 2.10 | ## UK | Does punching shear ever govern? If not remove it. | 15 | R | Discuss with Ad-hoc group. Results of the discussion: The rule should be kept in the EN 1993. The failure mode may govern, if the plate thickness is small. |
| 2.3.4 | ## UK | This is an out-dated method of construction that is never used – remove the whole of this clause. | 16 | P | Unless Rivets are used in other European Countries I suggest that we remove this clause. |
| Figure 2.14 | ## UK | This figure is very confusing and seems to be showing two different connection types. | 17 | A | Further clarification of the figure is required. |
| 2.3.6 (4) | ## UK | This clause is very badly worded. Please reword and make clear what is actually meant. | 18 | P | The clause is deleted form Part 1.8. Discuss the meaning of this clause with the ad-hoc group. |
| 2.4.1 | ## UK | This important clause seems to be tucked away at the end of the section where it will be difficult to find. | 19 | A | See reaction to comment 5. |
| 5.4.1 (2) | ## Finland | How is it possible, that the tension resistance of the net section of an angle with one bolt is greater than with two bolts? See the simple answer at the end of this table. | 20 | R | See UK comment no. 21 |
| 5.4.1 (2) | ## UK | Whilst it appear from your calculations that there is something wrong with the formulae in ENV 1993-1-1 Clause 6.5.2.3, I suspect that the capacity will be governed by the bolt strength, ie, one bolt would not provide adequate strength. For your example the bolt shear strength assuming a 12mm diameter bolt would only be about 28,000 N, so even the ‘reduced’ capacity of the two bolted angle section would not govern. It is possible that the strength of the member is calculated to be less with two bolts as eccentric moments are considered. I would be interested to see Jan Stark’s response, the author of the Clause, and agree with Colin Taylor’s response that you should forward your query to him | 21 |  | See above comment. |
| 2.1.2 | ## Belgium | This paragraph concerns the minimum end distance, the minimum value of e1. Only the item (1) is necessary and useful. Items (2) and (3) would be deleted : - e2 must not be mentioned in the chapter concerning the end distance; - under stress fluctuations, the design will determine the value of e1, but there is no reason to increase the minimum value ; - it is evident that e1 can be increased to provide an adequate bearing resistance, but this does not concern the minimum value. | 22 | A | See my reaction to Comment No. 1. |
| 2.1.3 | ## Belgium | The minimum value for e2 is 1.2 do and no more information must be given. e2 does not concern the bearing resistance. The resistance of the connected parts must be checked by design in the net section in the case of non preloaded bolts and in the gross section for preloaded bolts. | 23 | A | Delete clause (2). Also see reaction to comment (1). |
| 2.1.5 | ## Belgium | Only one sentence is necessary for the paragraph: “The minimum value for the spacings p1 and p2 is 2.2 do”. It is evident that the values can be increased in accordance to the results of the design but this does not concern the minimum values. | 24 | A | See reaction to Comment (1). |

| # Clause No | ## Country | # Remarks and comments | # Classification |  | # Proposed action |
| --- | --- | --- | --- | --- | --- |
| 2.1.8 (3) | ## Belgium | There is a complete confusion about this subject. The sentence mentioned was in all previous codes, but for connections in shear subjected to constant stress. We have no result of tests on connections with slotted holes subjected to stress fluctuations (please write everywhere “fluctuations” instead of “fluctations”). On the other hand, it is shown now that these connections can be used, under constant stress, if the direction of the load is perpendicular to the direction of the large diameter of the slotted holes. In this case, the bearing resistance is smaller than the one for holes with normal clearance: the coefficient 2.5 in the formula in table 2.2 must be reduced to 1.5. This coefficient must be specified somewhere, here or in table 2.2. | 25a) 25b) | R | The rule is deleted. To be checked. Agreed. Rule will be added. |
| Table 2.1 | ## Belgium | It must be mentioned somewhere that the high strength bolts are bolts of grade 8.8 or more. | 26 | AM | Only grades of bolts will be used in prEN 1993 |
| 2.2.2 (1) | ## Belgium | Non preloaded bolts can be used now in shear connections subjected to stress fluctuations, providing that the load is always acting in the same direction (see new draft of EN 1993-1-9 for the fatigue resistance). | 27 | A | Agreed. See new Part 1.9. All rules related to fatigue will be given in Part 1.9 |
| 2.2.3 (6) and (7) | ## Belgium | The two items do not concern the resistance of the bolts. The resistance of the connected parts must be checked separately. | 28 | AE | Agreed. See the new version of Part 1.8 |
| 2.2.3 (8) and table 2.3 | ## Belgium | Where are these considerations coming from? It is not useful to consider d instead of do and the classes of nominal bearing are not defined. | 29 | AM | Tables are deletd. |
| 2.2.4.1 (4) to (6) | ## Belgium | When oversize or slotted holes are used why to modify ks and  MS? Only ks could be adapted; it would be much simpler. | 30 | AM | See the new version of Part 1.8 |
| 2.2.4.3 (3) | ## Belgium | In class A: when using spray-metallized with aluminium, it must also be shown that the slip factor is not less than 0.5. The second and the third items could be in the same sentence. | 31 | R | All rules related to surfaces and their inspections will be moved into EN 1090-1 |
| 2.2.5 | ## Belgium | Same remarks about e2 and p2 as for bolts. | 32 | A | Agreed. All distences will be given in a common table. |
| General | ## Netherlands | The bilinear interaction is evaluated against tests based on the Annex Z procedure and I do not think that it is over-conservative. The bilinear interaction represents a design value for the capacity where a probability of less than 5% exists in having an actual capacity less that the predicted capacity. Combined with the respective -factor m2 = 1,25 the interaction formula results in design value. Looking to Fig. 4.17 of the “Guide to Design Criteria for Bolted and Riveted Joints” the given elliptic interaction curve represents (as I can see it) more or less the average value for the test results. | 33 | A | The bi-linear interaction formula will remain in the new version. |

General	Netherlands	Comparison of tensile strength according to EC3 and Guide at the intersection point of the bilinear equation:

- Assume that the shear plane passes through the threaded portion of an 8.8 bolt.

- From the bilinear we obtain:

Fv0,6FRd+Ft0,9⋅1,4FRd≤1⇒FvFRd=0,171forFt=0,9FRd

where : FRd = fubAsγMb

-  Substituting this result in the elliptic equation the result is:

Fv0,6FRd2+Ft0,9FRd2≤1⇒Ft=0,86FRdinsteadofFt=0,9FRd

Thus, the difference between the bilinear and elliptic interaction equation is 4,2%.  For design purposes this is in my opinion next to nothing.

Comments from Finland on Clause 5.4.1 (2)

According to ENV 1993-1-1 (clause 6.5.2.3) we get for the tension resistance of the net section:

For one bolt we get (See clause 6.5.2.3 and Table 6.5.1 in ENV 1993-1-1):

Aeff = 2(e2-0,5do)t = 2x(25-7)x5 = 180 mm2

Nu,Rd = Aefffu/M2 = 180x510/1,25 = 73440 N

For two bolts we get (See clause 6.5.2.3. and Table 6.5.1 in ENV 1993-1-1):

Aeff = β2 Anet = β2 (A-dot) = 0,4 x (480-14x5) = 164 mm2

Nu,Rd = Aefffu/M2 = 164x510/1,25 = 66912 N