| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
|  | CZ Wald | Give formula for resistance and stiffness for right moment only and not for right and left moment. The logic behind this is that also for beam-to-column joints the resistance is given for one direction only This has impact for 5.1.6.4 figure 5.6 5.2.8.2 figure 5.34 5.3.4 figure 5.36 | E | 1 | Proposal has certain logic The consequences are investigated The document has been changes in several places |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
|  | CZ Wald | Table with effective length for base plate is missing | R | 1 | For the effective length is referred to the behaviour of an end plate in bending, see clause 5.2.6.11 |

|  | NOR Aasen | Reinforcement should comply with EC2 | A | 1 | To be addressed in part 1.1 Furthermore, a clause is added saying that it is a prerequisite of the rules that anchor bolts yield before tear out of the concrete, see 5.2.6.12(7) |
| --- | --- | --- | --- | --- | --- |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.1.1(10) | UK Moore | Does this mean the columns | A | 1 | This clause is taken from Annex J and doesn't reflect the scope of part 1.8, where Annex L and chapter 6 also are included. To be discussed in ad-hoc group |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.1.6.4 (2) | CZ Wald | Include "if not calculated more accurately, see 5.1.7.4(4) | AM | 1 | This clause will be rephrased: "should" will be replaced by "may" |
| 5.1.6.4 (2) | UK Moore | Please note that all bolts must be within the width of the column flange. | A | 1 | This is correct and need to be clarified. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.1.6.4 (3) | UK Moore | How is the stress block under the plate calculated | R | 1 | see 5.2.3 |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.1.6.4 (5) | CZ Wald | Reference to 5.1.7.4(4) | AM | 1 | Clause 5.1.7.4(4) will be deleted, because it is over complete. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.1.7.4 (4) | CZ Wald | Cut "taking the tolerances............into account" | AM | 1 | Clause 5.1.7.4(4) will be deleted, because it is over complete. |
| 5.1.7.4 (4) | FI Kouhi | Proposal made as included in 19 October draft part 1.8 | R | 2 | Clause 5.1.7.4(4) will be deleted, because it is over complete. |
| 5.1.7.4 (4) | BE Jaspart | The contents of this clause should be checked again after completion of 5.1.6.4 | A | 3 | To be done |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.1.7.4 (6) | FI Kouhi | Proposal made as included in 19 October draft part 1.8 | M | 1 | There are many thoughts in this clause: - shear can be taken by special shear connectors - shear can be taken by a combination of friction and holding down bolts - the surrounding part of the foundation should be able to take the shear load too - in case of holding down bolts, concrete crushing due to shear should be considered and checked according to Eurocode 2 Actions: - to split up in different clauses and - to clarify that holding down bolts and shear trough friction can be combined and - that concrete crushing due to shear should be checked according to EC2. |
| 5.1.7.4(6) | UK Moore | Does rupture mean crushing? | A | 2 | To be edited |
| 5.1.7.4 (6) | BE Jaspart | The contents of this clause should be checked again after completion of 5.2.1.2(7) | A | 3 | To be done |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.1.2 (7) | CZ Wald | Does not correspond to 5.1.7.4(6) Separate (7) friction resistance and (8) anchor bolt resistance Mention the assumption of hole clearances, see prEN1090 | A A A | 1 | Clause 5.1.7.4(6) will be improved to clarify: ".........is provided by one or more of the following: ..........." Proposal will be made Proposal will be made |
| 5.2.1.2 (7) | FI Kouhi | Proposal to give for "steel grade" | P | 3 |  |
| 5.2.1.2 (7) | UK Moore | No check on bearing of the concrete/grout | A | 3 | The grout is not assumed to bear shear loads. For the crushing of the concrete in shear, clause 5.1.7.4(6) will be modified and give a reference to EC2 |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.1.3 (5) | B Jaspart | Quid for column basis where there is an axial force | A | 1 | Improve the text to refer to beam-to-column joints and beam splices |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.2.1 (9) | B Jaspart | This clause should be extended for anchor bolts | R | 1 | Agreed, the clause is valid for bolts in general, but since the behaviour of a base plate in bending is treated as an end plate in bending, this clause is also valid for base plates. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.2.1 (8) | UK Why no prying for mode 1? | Why no prying for mode 1 | R | 1 | In case of "long" anchor bolts, the bolts may elongate such that no prying occurs, even in mode 1. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.2.3 (1) | B Jaspart | Should such an reinforcement not be applicable to base plates where washer plates are already often used. | R | 1 | In principle, this is a good question. The rules given are however not applicable to this case, since it is assumed that the T-stub length of the backing plate is equal to or more than that of the flange. It is suggested to leave this matter and the modification of rules to the engineering judgement. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.3 | CZ Wald | Turn horizontally to be compression in all code on right. | E | 1 | To be edited |
| 5.2.3 | CZ Wald | Move fig. 5.27 to this chapter | E | 2 | To be edited |
| 5.2.3 | CZ Wald | in (a) size of fonts | E | 3 | To be edited in clause number (10). In clause number (11) is also a typing error b1 stead b1. |
| 5.2.3 | FI Kouhi | Limit on kj < 5 | R | 4 | This is not needed, because kj cannot exceed 5. |
| 5.2.3 | FI Kouhi | Meaning on "short and long projection" unclear | E | 5 | To be discussed in ad-hoc group |
| 5.2.3 | UK Moore | Fig 5.27 seems wrong | E | 6 | Indeed, it is at the wrong place, see comment 2 |
| 5.2.3 | LU Braham | Comment 36 The aim of using a T-stub model in compression is not clear. | R | 7 | The idea of introducing a T-stub concept is that the compression envelop under an I section can be subdivided in three areas: two under the flanges and one under the web. For sake of simplicity, when calculating the resistance of a column base under compression and bending moment, the web may be neglected. In the case of full compression, it is easy to take advantage of the web portion. |
| 5.2.3 | LU Braham | Comment 36 5.2.3(6) and (7) are difficult to understand | E | 8 | What is the question |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.6.9 | CZ Wald | reorganize as (check for crushing in concentrated compression in 5.2.3 (concrete slab in EC2) | R | 1 | The logic of the document is to list all components in 5.2.6. This is just one of them, and the component is treated like a T-stub, as for instance column flange in bending., see 5.2.6.4.2 The punching of the concrete slab is covered under "concrete in compression" |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.6.12 | CZ Wald | cut (1) (2) (3) as they are already treated in 5.1.7.4 | E | 1 | Agreed, to be done in editing |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.6.12 (3) | LU Braham | Comment 11 In paragraph 5.2.6.12(3) it is indicated that one should "take into account the tolerance on the positions of the holding down bolts". This is not a topic of a design code to make such a comment; probably the tolerances on holding down bolts should be reviewed. | A T | 1 | To delete this clause in part 1.8 To clarify tolerances on holding down bolts in EN 1090. |
| 5.2.6.12 | UK Moore | What is meant by hook bolt |  | 2 | Question unclear |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.3.1 | CZ Wald | table 5.6, comma in 2,7 and 3,1 | E | 1 | Agreed, to be done in editing |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.2.3 | NOR Aasen | Why is the elastic resistance of a plate used for the resistance of a column base plate in compression: 1/6 t2 leff fy and not 1/4t2 leff fy | R | 2 | The behaviour of the area under the base plate is modelled by an equivalent stress block under the flange of web of a column. The width of this stress block is based on the elastic resistance of the plate. Assume now, that it is calculated by the plastic resistance, then a plastic hinge will form in the plate just next to the welds connecting the plate just next to the flange or web. The effective area will reduce to something like the flange thickness + 2 times the weld throat thickness. This is not the intention of the model. That is why it is assumed that the plate remains elastic, so the stress area can develop without excessive deformations. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.3.2 | CZ Wald | (15) Errors, OK see COST C1 page 2.32, 233, 234 (16) Errors, OK see COST C1 page 2.32 | A | 1 | Agreed, "prying" in formula (5.54b) and (5.55b) will be changed in "no-prying". |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 5.4.1 | UK Moore | What about ground / structure interaction | X | 1 | Ground/structure interaction should be considered, but is out of scope of part 1.8 |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 7.2.2.3 (2) | B Jaspart | For column basis another criterion has also be used in addition to the one expressed in the clause: 10% on lateral sway displacement. | A | 1 | To implement. |

| Clause No. | Country | Remarks and comments | Classif. |  | Proposed action |
| --- | --- | --- | --- | --- | --- |
| 7.2.3 | B Jaspart | The clauses are too restrictive, since the should be applicable to all joints. | A | 1 | Delete "beam-to-column joint" and replace by "joint" in clause 7.2.3.1(1) |