##### 7 STEEL-CONCRETE CONNECTIONS

# 7.1 Column Bases

The EN 1993-1-8 [Eurocode 3, 2001] includes rules to determine the strength and stiffness the column bases, which is the system of column, of base plate and of holding down assembly.  The end plates are designed preferable unstiffened, but the design procedure for stiffened base plates may be utilize.  The Procedure is applicable for columns of open and of closed cross sections [Wald at al., 2000].  Further column base details may be adopted, including embedding the lower portion of column into a pocket in the foundation, or the use of base plates strengthened by additional horizontal steel members.  Foundations themselves are supported by the sub-structure.  The foundation may be supported directly on the existing ground, or may be supported by piles, or the foundation may be part of a slab.  The influence of the support to the foundation, which may be considerable in certain ground conditions, is not covered in the EN 1993-1-8 [Eurocode 3, 2001].

Fig. 7.1   Typical column base assembly of column, base plate and holding down bolts, a) bolts inside the base plate, b) bolts outside the base plate (an optional stiffener), the components for prediction of stiffness and resistance

Traditional approaches to the design of the pinned bases predict the base plate thickness stiff enough to be modelled as all rigid to ensure the uniform stress in compression.  The traditional design of moment-resisting bases involves also an elastic analysis, based on the assumption that plane sections remain plane.  By solving equilibrium equations, the maximum stress in the concrete (based on a triangular distribution of stress), the extent of the stress block and the tension in the holding down assemblies may be determined.  Whilst this procedure has proved satisfactory in service over many years, the approach ignores the flexibility of the base plate in bending (even if they are strengthening by stiffener), the holding down assemblies and the concrete.

The concept adopted in EN 1993-1-8 [Eurocode 3, 2001] is the transfer of the flexible base plate into the effective rigid plate allowing the strength of concrete in concentrated compression, which is crushing of the concrete surface.  The plastic distribution of the internal forces is applied.  The component method, the same as for the beam-to-column joint prediction, is used to allow also calculation of stiffness.  The component approach involves identifying each of the important features in the base connection and determining the strength and stiffness of each of these ‘components’.  The components are then ‘assembled’ to produce a model of the complete arrangement.

The resistance of column base is included in EN 1993-1-8 [Eurocode 3, 2001] in Chapter 6.2.6. The stiffness column bases is introduced in Chapter 6.3.4.  Details of the components in a column base connection are described as well, namely: the compression side - the concrete in compression and the flexure of the base plate (Chapter 6.3.2), the column flange and web in compression (Chapter 6.2.4.7), the tension side - the holding-down assemblies in tension and the flexure of the base plate (Chapter 6.2.4.12), the transfer of horizontal shear (Chapter 6.2.1.2).  The classification boundaries for column base stiffness are included in Chapter 3.2.2.5.

# Elastic resistance of a base plate

Why do you calculate the resistance of a base plate based on the elastic 1/6 t2 and not the plastic 1/4 t 2?

__________________________________________________________________________

The formula for calculation of the effective bearing area under the flexible base plate around the column cross section is based on estimation of the effective width c.  It secures that the yield strength of base plate is not exceed in the compression zone but limits also model of concentrated stress under the flexible base plate by restricting the deformations of plate into the elastic range only [Bijlaard, 1982].  Elastic bending moment resistance of the base plate per unit length should be taken as

M′=16 t2 fy	(7.1)

and the bending moment per unit length on the base plate acting as a cantilever of span c is, see Fig. 7.1:

M′=12 fj c2.	(7.2)

When these moments are equal, the bending moment resistance is reached and the formula evaluating c can be obtained

12 fj c2=16 t2 fy	(7.3)

as

c=t fy3 γM0 fj	 (7.4)

Fig. 7.1 The engineering model of the base plate as a cantilever

Fig. 7.2  2 D finite element model of base plate T stub ant concrete block in compression, the net, the deformed mesh, the major stresses in concrete [Wald, Banitopoulos, 1998]

# Calculation of base plate resistance with low quality grout

In Annex L, the joint coefficient βj is taken as 2/3 when the grout has at least 20% of the characteristic strength of the concrete foundation. What value should be taken when the strength of the grout is smaller?

___________________________________________________________________________

The problem of low quality grout was studied by experimentally and numerically.  It was found, that the grout layer of limited thickness does not affect the resistance of concrete in bearing.  It is expected that the layer under three-dimensional boundary conditions, the mortal between the concrete and the base plate, behaves similar to liquid.  All test ware carried with real grouts of same resistance. These boundaries of knowledge are introduced in standard.  The lower resistance of mortal (just a sand) may cause the changes of the working diagram of the layer material under the repeated loading.

Most of the used mortals have higher resistance compare to the concrete block [Stark, Bijlaard, 1988]. In such case the layer may be neglected or taken into account by checking its bearing resistance as well as concrete block resistance with distribution of normal stresses under the effective rigid plate in angle of 45°, see Fig. 7.3.

Fig. 7.3   The stress distribution in the grout

# Comparison of concrete strength calculation according to EC2 and EC3

It seems the results of calculation of bearing strength of column base fj are the same as calculation according to EC2.

According to 5.4.8.1 of ENV 1992 is:

_____

FRdu = Aco * fcd * Ac1/Ac0     3,3 fcd * Aco.

According to EC3 the maximum value for kj is 5,0. For this value we get the value of fj = (2/3) * 5 * fcd = 3,33 fcd.

The result is the same as given in the EC2, however the methods are different in EC3 and EC2. Could you find any background information about this?

___________________________________________________________________________

The Eurocodes 2 and 3 are solving of the same problem of the bearing resistance of concrete under the steel plate.  The bearing resistance is limited by the crashing of the concrete.  The technical literature concerned with the bearing strength of the concrete block loaded through a plate may be treated in two broad categories.  Firstly, investigations focused on the bearing stress of rigid plates, most were concerned the prestressed tendons.  Secondly, studies were concentrated on flexible plates loaded by the column cross section due to an only portion of the plate.

The experimental and analytical models are including the ratio of concrete strength to plate area, relative concrete depth, the location of the plate on the concrete foundation and the effects of reinforcement.  The result of these studies on foundations with punch loading and fully loaded plates offer qualitative information on the behaviour of base plate foundations where the plate is only partially loaded by the column.  Failure occurs when an inverted pyramid forms under the plate.  The application of limit state analysis on concrete can include the three-dimensional behaviour of materials, plastification and cracking.  Experimental studies [Shelson, 1957; Hawkins, 1968, DeWolf, 1978] led to the development of an appropriate model for column base bearing stress estimation that was adopted into the current codes.  The separate check of the concrete block itself is necessary to provide to check the shear resistance of the concrete block as well as the bending or punching shear resistance according to the concrete block geometry detailing.

The proposed model is validated against the tests.  50 tests in total were examined in this part of study to check the concrete bearing resistance [DeWolf, 1978; Hawkins, 1968].  The test specimens consist of a concrete cube of size from 150 to 330 mm with centric load acting through a steel plate.  The size of the concrete block, the size and thickness of the steel plate and the concrete strength are the main variables.  Figure 7.5 shows the relationship between the slenderness of the base plate, expressed as a ratio of the base plate thickness to the edge distance and the relative bearing resistance. The design approach given in Eurocode 3 is in agreement with the test results, but conservative. The bearing capacity of test specimens at concrete failure is in the range from 1,4 to 2,5 times the capacity calculated according to Eurocode 3 with an average value of 1,75.

Fig. 7.4  Relative bearing resistance-base plate slenderness relationship, experiments [DeWolf, 1978, and Hawkins, 1968]

The influence of a flexible plate was solved by replacing the equivalent rigid plate [Stockwell, 1975]. This reasoning is based on recognition that uniform bearing pressure is unrealistic and that maximum pressure would logically follow the profile shape. This simple practical method was modified and checked against the experimental results, [Bijlaard, 1982; Murray, 1983]. Eurocode 3, Annex L [Eurocode 3, 1992] adopted this method in conservative form suitable for standardisation using an estimate including the dimensions of the concrete block cross-section and its height. It was also found [DeWolf and Sarisley, 1980; Wald, 1993] that the bearing stress increases with larger eccentricity of normal force. In this case is the base plate in larger contact with the concrete block due to its bending. In case, when the distance between the plate edge and the block edge is fixed and the eccentricity is increased, the contact area is reduced and the value of bearing stress increases. In case of the crushing of the concrete surface under the rigid edge is necessary to apply the theory of damage. These cases are unacceptable from design point of view and are determining the boundaries of above described analysis.

The influence of the concrete strength is shown on Figure 7.6, where is shown the validation of the proposal based on proposal tw + 2 c. A set of 16 tests with similar geometry and material properties was used in this diagram from the set of tests [Hawkins, 1968]. The only variable was the concrete strength of 19, 31 and 42 MPa.

Fig. 7.5  Concrete strength - ultimate load capacity relationship [Hawkins, 196])

# Stress concentration factor kj for column bases

Please, provide background documentation to justify using value of fj, which can lead to values of fj more than 10 times higher than the characteristic strength of the grout. According to Annex L of Eurocode 3, the maximum value for kj is 5,0 for a square base plate. For this maximum value we get the maximum value of fj = (2/3) * 5 * fcd = 3,33 fcd. It is recommended to use joint coefficient βj = 2/3 when the characteristic strength of the grout is not less than 0,2 times the characteristic strength of the concrete, therefore the lowest strength of the grout is fcd.g = 3,33 * fcd / 0,2 = 16,66 fcd.

___________________________________________________________________________

The resistance of the grout and the concrete block in compression is limited by the crushing of the grout or concrete under the flexible base plate.  In engineering models, the flexible base plate is transferred to an equivalent rigid plate round the column cross section, see Fig. 7.7, [Eurocode 3, 2001].  The calculation of the bearing resistance Fc.Rd under the base plate is based on the evaluation of the joint concentration factor kj.  The concrete bearing resistance fj is calculated from

a1=min a+2 ar5 aa+h, a1≥a	(7.5a)

b1=min b+2 br5 bb+h, b1≥b	(7.5b)

kj =   a1  b1a  b    	(7.6)

fj = βkj  fckgc   	(7.7)

c= t fy3 fjgM0	(7.8)

Fc.Rd=Aefffj	(7.9)

In these formulas is fck the characteristic value of the concrete compressive cylinder strength, c the partial safety factor for concrete and M0 the partial safety factor for steel.  The effective area Aeff round the part of the column cross – section, which is in compression is described in Fig. 7.6.

The grout quality and thickness is introduced by the joint coefficient . For j = 2/3, it is expected that the grout characteristic strength fc.g is not less than 0,2 times the characteristic strength of the concrete foundation fc ( fc.g  0,2 ) and the thickness of the grout is tg  0,2 min (a ; b).  In cases of a different grout quality or higher thickness of the grout tg  0,2 min (a ; b), it is necessary to check the grout separately.  In this case the three-dimensional conditions of grout can be treated similar to concrete block.

Fig. 7.6  Local deformation of the concrete block, the effective area under the flexible plate

The concrete bearing resistance in joint express the case, where the concrete is loaded in compression under 3D conditions. In this case the reached resistance was experimentally find higher compare to simple compression about 6,25 higher. This is reflected in Eurocode rules by joint factor kj with the maximum value 5,0 for a square base plate.

The problem of grout is different, we may take the layer of grout into calculation or we may apply the knowledge in Eurocode. The thin layer of grout is not affecting the column base resistance.

The both effects are separate problem and may not be mixed by increasing the resistance.

# Effective length of base plate T-stub

Can the table with effective length of end-plate connections be used also for base plates?

__________________________________________________________________________

The tables may be used but the modes with prying and without prying needs to be distinguished.  The effective length of the T-stub in tension is affected by the failure mode of the T stub.  The anchor bolts are rather longer and the base plates mostly thicker compare to typical configuration of end plates in the beam to column connections.

Fig. 7.7  T-stub in case of lost of the contact to the concrete block

The boundaries between the praying and not prying modes may be found form the assumption of n = 1,25 m, see Fig. 7.7,

Lb.lim=8,82 m3 Asℓeff t3  Lb,	(7.10)

where As is the bolts area and Lb is the holding down bolt free length, see Fig. 7.7.  For the bolts embedded into concrete this length Lb may be assumed as free length Lbf and effective free length of embedded part as Lbe  8 d, e.g. Lb = Lbf  + Lbe, see [Wald, 1999].  For the bolt length Lb longer to Lb.lim no prying occurs.

Fig. 7.8  Free length of bolts in case of embedded into concrete block

Fig. 7.9  Failure mode 1* for the T-stubs of column base

Fig. 7.10  Effective lengths of T-stub of base plate with bolts inside the column flanges

Tab. 7.1  Effective lengths of T-stub of base plate with bolts inside the column flanges

| prying occurs ℓ1=2 α m−4 m+1,25 e ℓ2=2 π m ℓeff,1=min ℓ1; ℓ2 ℓeff,2 =ℓ1 | no prying ℓ1=2 α m−4 m+1,25 e ℓ2=4 π m ℓeff,1=min ℓ1; ℓ2 ℓeff,2 =ℓ1 |
| --- | --- |

Fig. 7.11  Effective lengths of T-stub of base plate with bolts outside the column flange

text to be completed

Tab. 7.2  Effective lengths of T-stub of base plate with bolts outside the column flange

| prying occurs l1 = 4 mx +1,25 ex l2 = 2  mx l3 = 0,5 bp l4 = 0,5 w + 2 mx + 0,625 ex l5 = e + 2 mx + 0,625 ex l6 = π mx+2 e ℓ7 = π mx+p ℓeff.1=min ℓ1; ℓ2; ℓ3; ℓ4; ℓ5; ℓ6; ℓ7 ℓeff.2=min ℓ1; ℓ3; ℓ4; ℓ5 | no prying l1 = 4 mx +1,25 ex l2 = 4 mx l3 = 0,5 bp l4 = 0,5 w + 2 mx + 0,625 ex l5 = e + 2 mx + 0,625 ex l6 = 2 π mx+4 e ℓ7 = 2 (π mx+p) ℓeff.1=min ℓ1; ℓ2; ℓ3; ℓ4; ℓ5; ℓ6; ℓ7 ℓeff.2=min ℓ1; ℓ3; ℓ4; ℓ5 |
| --- | --- |

# Effective length of base plate with bolts outside the column flange

The tables for calculating of effective width of a T-stub include only cases where all bolts are placed within the width of the beam flange. When this table is applied to base plates, the bolts are often outside the column flange. Can these formulas still be used?

___________________________________________________________________________

The yield pattern of cases where all bolts are placed outside the width of the beam flange where studied by [Wald at al., 2000].  The application of the tables for beam to column joints needs to be modified.  One more pattern may occur.

Fig. 7.12  The base plate geometry a), assumption of the range of effective length of T-stub for base plate b)

The effective length of the base plate T-stub can be determined by the yield line method.  The yield line is a straight line, and this line is perpendicular to a line, that pass through the bolt and the corner of the plate.  represents the angle of the yield line with the edge and c the minimal distance between the corner of the plate and the yield line.  The following relations can be obtained

tanα=xy,	(7.11)

where x and y are the variable coordinates of the bolt. For the design of the parameter c, we use the work method of the yield line theory. The internal work

Wi=n θj; muj; ℓj=mpl 1y x+1x y.	(7.12)

The external work

We=Pu Δ=Fpl Δ.	(7.13)

 represents the deformation of the plate in the bolt position, see Fig. 4.

Δ1=dc=x2+y2c,	(7.14)

x2+y2c Fpl=mpl xy+yx,	(7.15)

ℓeff=c m4 x2+y2x y,	(7.16)

For the resistance can be derived

Fpl=c mpl x2+y2x y,	(7.17)

∂ Fpl∂ c=mpl x2+y2xy=cst	(7.18)

Five cases may be observed for the yield lines round by the corner of the column, see Tab. 7.3 from [Wald at al, 2000], if are taken into account the modes without the contact of the edge of base plate to the concrete surfaces, e.j. in no prying cases.

Tab. 7.3  The calculation of the effective length of a T-stub per bolt, Case 1 to 3

| Case 1 | Case 2 | Case 3 |
| --- | --- | --- |
| Wext=Fpl δ | Wext=Fpl δ δ=a−ac−2 eaa−ac | Wext=Fpl δ δ=b−bc2+a−ac2−2 ea2+eb2b−bc2+a−ac2 |
| Wint=4 π mpl δ | Wint=mpl ba−ac | Wint=mpl eaeb+ebea |
| Fpl=4 π mpl | Fpl=mpl ba−ac−2 ea | Fpl=mplδ eaeb+ebea |
| m=a−ac2−ea |  |  |
| ℓeff.1=π m | ℓeff.2=b4 | ℓeff.3=a−ac2+b−bc28 eaeb+ebea |

The Case 4 and Case 5 are similar to 2 and 1 respectively. The results of prediction of effective lengths per anchor bolt are summarised in Tab. 2. The results of the FE simulation is shown at Fig. 13 in the case no prying.

Fig. 13  FE mesh of simulation, the different yield patterns under the moving of the anchorage round the base plate corner

# Slip factor between steel and concrete

What is the slip factor between steel and concrete?

___________________________________________________________________________

In the EN 1993-1-8 Cl. 6.2.1.2 [Eurocode 3, 2001] is included the coefficient of friction between base plate and ground layer Cf.d = 0,20 for the sand-cement mortar and Cf.d = 0,30 for the special grout.

In the CEB Guide [CEB, 1997] is for a thin layer or grout, thickness less than 3 mm, used a friction coefficient is used of 0,4. In this case is recommended the partial safety factor for the ultimate limit state design as Mf = 1,5.

# Transfer of shear forces by anchor bolts

Can anchor bolts be used to transfer horizontal forces into concrete foundation?

___________________________________________________________________________

The use of anchor bolts to transfer the shear resistance forces is approved by practice, in USA for example [DeWolf and Ricker 1990], by many years.  The boltholes in base plate need to have a clearance in tolerances as recommended by EN 1990.  The most conservative approach is summarized in CEB document.  It is expected that the anchor bolt acts as cantilever of length of mortal thickness plus 0,5 d in the case of post installed or cast-in –place anchors. The cantilever is guided in base plate, its length is reduced to L/2, see Fig. 7.14.

The advanced model of resistance to shear forces is described in EN 1993-1-8.  The resistance is based on experimental and analytical work [Bouwman at al., 1989]. The model expects the deformation of the anchor bolt and the development of the membrane effect by the tension force bolt and the compressed force in the mortal.  The design procedure is simplified for practical application, see EN 1993-1-8 Cl. 6.2.1.2 [Eurocode 3, 2001], see Fig. 7.15.

Fig. 7.14  Model of anchor bolt in bending

Fig. 7.15  The shear behavior of an anchor bolt, the resistance in tension

# Transfer of shear forces by friction and anchor bolts

Is it safe adding the friction resistance with the bearing resistances of all the anchor bolts, as clearances in anchor boltholes are large and with uncontrolled distribution.

___________________________________________________________________________

The advanced model of resistance to shear forces used in EN 1993-1-8 Cl. 6.2.1.2 [Eurocode 3, 2001] is based on the evaluation of resistance based on deformed shape.  The bolt resistance in bending is smaller compare to the tensile resistance.  This allows for deformable ductile bolts to use the post bending resistance on deformed shape, see Fig. 7.16.  The shear resistance than consist of the friction resistance and of all bolts reduced tensile resistance.

Fv.Rd=Ft.Rd+n Fvb.Rd	(7.19)

The resistance of the bolts in bearing in the concrete block and the bearing resistance of the bolts in base plate need to be checked separately.  Only the anchor bolts in the compressed part of the base plate may be used for the transfer of shear forces.

Fig. 7.16   The friction resistance and in tension resistance of anchor bolt

# Rules for anchorage of holding down bolts

The proper anchorage is the most important criteria for appropriate design of holding down bolts but it is not dealt within Annex L. Where are given the rules for anchorage?

___________________________________________________________________________

The models of design resistance of anchoring compatible with Eurocode were published in CEB Guide [CEB, 1997] and in contribution by [Eligehausen 1990]. The required verification for a single anchor bolt is:

steel failure

NSd≤Na.Rd≤NRd.S=fybAsgMb	(7.20)

pullout failure

NSd≤Na.Rd≤NRd.p=NRk.pgMp,	(7.21)

concrete cone failure

NSd≤Na.Rd≤NRd.c=NRk.cgMc	(7.22)

and splitting failure of the concrete

NSd≤NRd.sp=NRk.spgMsp.	(7.23)

Similar verification is required for anchor group Nsdh.

The rules of EUROCODE 3, Tab. 6.5.3, [Eurocode 3, 1992] on bolts tension resistance are satisfied for all steel grades used for anchor bolts

Fv.Rd≤βb0,9fubAsgMb,	(7.24)

where fyb  is the yield tensile strength of bolt, Mb the partial safety factor for the bolts, fub the ultimate tensile strength of the bolt, b the cut tread reduction factor, if applicable b = 0,85.

Different types of anchoring are used, hooked bars for light anchoring, the cast-in-place headed anchors and bounded anchors to drilled holes. The expensive anchoring to grillage beams embedded in concrete are designed for large frames, see Fig. 7.17.

### Fig. 7.17  The geometry of the In-situ-cast headed anchor bolt

The calculation of design resistance of fastenings with cast-in-situ headed anchor bolts loaded by tension force is included in the following part.

The pullout failure design resistance can be obtained as

NRd.p=pkAhgMp,	(7.25)

where pk is taken in non-cracked concrete as

pk = 11,0 fck	(7.26)

and Ah is the bearing area of the head, for circular head can be written

Ah =  (dh2 - d2) / 4	(7.27)

The concrete cone failure design resistance is given as

NRd.c=NRd.c0Ac.NAc.N0Ys.NYec.NYre.NYucr.N,	(7.28)

where

NRk.c0=k1fck0,5hef1,5gMc	(7.29)

is the characteristic resistance of a single fastener. The coefficient k1 could be taken for non cracked concrete as

k1=11,0Nmm.	(7.30)

The geometric effect of spacing p and edge distance e is included in calculation of the area of the cone, see Fig. , as

Ac.N0
            =pcr.N2,	(7.31)

Ac.N=pcr.N+p1pcr.N+p2,	(7.32)

for example for Fig. a,b, or

Ac.N=e+0,5pcr.Npcr.N	(7.33)

for Fig. c. It is possible to take approximately

pcr.N@2,0ecr.N@3,0hef	(7.34)

Fig. 7.18  An idealised concrete cone, individual anchor a), anchor group b), single anchor at edge c)

The disturbance of the stress distribution in concrete could be introduced by parameter

Ys.N=0,7+0,3eecr.N≤1.	(7.35)

The parameter ec.N  takes into account the group effect. Parameter re.N  is used for small embedded depths (hef  100 mm). The resistance is increased in non-cracked concrete by parameter Yure.N=1,4.

The splitting failure for the in-situ-cast anchors is prevented if the concrete is reinforced or by limiting:

spacing

pmin = 5 dh  50 mm,	(7.36)

edge distances

emin = 3 dh  50 mm,	(7.37)

and height of the concrete block

hmin = hef + th + c,	(7.38)

where

th	is thickness of anchor head and

c	required concrete cover for reinforcement.

For fastenings with an edge distance e > 0,5 hef  in all directions, a check of the characteristic blow-out resistance may be omitted.

The detailed complex description of the design resistance of different types of fastenings loaded by tensile force, by shear force and by combined tension and shear forces is included in CEB Guide [CEB, 1994].

# Base plate of circular hollow section

Are there any guidelines how to handle the T-stub of a circular hollow column section on a rectangular base plate?

___________________________________________________________________________

to be completed

# Yield strength of hooked anchor bolts

Why is yield strength of anchor bolt limited to 300 MPa in case of anchorage with hook (paragraph 5.2.6.12 in EC3 part 1.8)?

___________________________________________________________________________

to be completed

7.2 Beam-to-Column Steel-Concrete Connections

to be completed

Symbols

| a | length of the base plate |
| --- | --- |
| a1 | effective length of the foundation |
| b | width of the base plate |
| bc | width of the column |
| bw | width of the washer |
| b1 | effective width of the foundation |
| c, ci | effective width |
| c | required concrete cover for reinforcement |
| d | diameter of the bolt |
| dh | diameter of anchor head |
| dw | diameter of washer |
| d0 | diameter of the bolt hole |
| e, ea, eb | deviation, distances |
| fcd | design value of concrete compressive cylinder strength fcd = fck / c |
| fck | characteristic value of concrete compressive cylinder strength |
| ft | effective stress of the tension part |
| fj | concrete bearing strength |
| fjh | concrete bearing strength in horizontal direction |
| fy | yield stress of steel |
| fyb | yield stress of the bolt |
| fu | ultimate tensile strength |
| fub | ultimate strength of the bolt |
| g | distance |
| h | height of the foundation |
| hc | height of column cross section |
| kj | concentration factor |
| ki | component stiffness |
| m | distance from the bolt axes to the weld edge |
| p | pitch |
| q | load |
| rb | lever arm of bolt force |
| rc | lever arm of concrete force |
| s | elongation |
| t | thickness of the base plate |
| tf | thickness of the flange |
| th | thickness of anchor head |
| tp | limiting thickness of base plate for prying |
| tw | thickness of the column web |
| twa | thickness of the washer |
| x, y, z | axes, distance |
| A | area |
| Ab | total area of the bolt, untreated part |
| Ac | area of the column |
| Aeff | effective area |
| Ah | bearing area of the bolt head |
| As | net area of the bolt, in tread |
| E | Young’s modulus, Young’s modulus of steel |
| Ec | Young’s modulus of concrete |
| F | force |
| I | second moment of inertia |
| L | length |
| Lb | length of the untreated part of the anchor bolt |
| Lbe | effective length of the anchor bolt |
| Leq | equivalent length of the anchor bolt |
| Ls | length of the treated part of the anchor bolt |
| Leff | effective length |
| M | bending moment |
| M´ | bending moment of unique width |
| Q | prying force |
| Sj | joint stiffness |
| Sj.ini | initial stiffness |
| S | relative stiffness |
| V | shear force |
| W | section modulus |
| a | coefficient |
|  | relative thickness of the base plate |
| j | joint coefficient |
| w | correlation factor |
|  | rotation |
| i | partial safety factor |
|  | ratio of the base plate dimensions, a / b |
|  | stress |
| T | stress in the tension part |
|  | normal stress perpendicular to the throat |
| II | shear stress, in the plane of the throat, parallel to the axis of the weld |
|  | shear stress, in the plane of the throat, perpendicular to the axis of the weld |

Subscripts

| b | bolt |
| --- | --- |
| c | concrete, column |
| k | characteristic |
| cr | critical |
| eff | effective |
| eq | equivalent |
| f | flange |
| ge | free length |
| h | horizontally, bolt head |
| n | non-sway |
| p | plate, effective |
| pl | plastic |
| s | sway, treated part |
| w | web |
| wa | washer |
| we | weld |
| Rd | resistance, design |
| Sd | internal, design |

References

[Eurocode 3, 2001] EN 1993-1-8. Eurocode 3: Design of Steel Structures, Part 1.8: Design of Joints, European Norm, CEN, Brussels 2001.

[Eurocode 2, 1992] ENV 1992-1-1: Design of Concrete Structures, Eurocode 2, European Prenorm, CEN, Brussels 1992.

[Eurocode 3, 1992] ENV 1993-1-1: Design of Steel Structures, Eurocode 3, European Prenorm, CEN, Brussels 1992.

[Eurocode 3-A2, 2001] ENV 1993-1-1-A2: Design of Steel Structures, Annex J, Joint Design, European Prenorm, CEN, Brussels1992.

[CEB, 1997] Design of Fastenings in Concrete, Design guide, CEB, Thomas Telford Services Ltd, London 1997, p. 83, ISBN 0 7277 2558 0.

[CEB, 1994] Fastenings to Concrete and Masonry Structures, State of the Art Report, CEB, Thomas Telford Services Ltd, London 1994, p. 248, ISBN 0 7277 1937 8.

[Bijlaard, 1982] Bijlaard F.S.K.: Rekenregels voor het ontwerpen van kolomvoetplaten en experimentele verificatie, Rapport No. BI-81-51/63.4.3410, IBBC-TNO, Delft 1982.

[DeWolf and Ricker, 1990] Dewolf J.T., Ricker D.T.: Column Base Plates, Steel Design Guide, Series 1, AISC, Chicago 1990.

[DeWolf and Sarisley, 1980] DeWolf J.T. and Sarisley E.F.: Column Base Plates with Axial Loads and Moments, Journal of the Structural Division ASCE, Vol. 106, No. ST11, 1980, pp. 2167-2185.

[DeWolf, 1978] DeWolf J.T.: Axially Loaded Column Base Plates, Journal of the Structural Division ASCE, Vol. 104, No. ST4, 1978, pp. 781-794.

[Eligehausen, 1990] Eligehausen R.:  Design of fastenings in concrete using partial safety factors, in German, Bauingenier 1990, No. 65, pp. 295-305.

[Hawkins, 1968] Hawkins N.M.: The bearing strength of concrete loaded through rigid plates, Magazine of Concrete Research, Vol. 20, No. 63, March 1968, pp. 31-40.

[Hawkins, 1968] Hawkins N.M.: The bearing strength of concrete loaded through flexible plates, Magazine of Concrete Research, Vol. 20, No. 63, June 1968, pp. 95-102.

[Murray, 1983] Murray T.M.: Design of Lightly Loaded Steel Column Base Plates, Engineering Journal AISC, Vol. 20, 1983, pp. 143-152.

[Nakashima, 1998] Nakashima S.: Effects of floors on mechanical behaviours of Steel column bases, in Proceedings of Nordic Steel Construction Conference 98, Oslo 1998, Vol. 2., pp. 467 - 474.

[Shelson, 1957] Shelson W.: Bearing Capacity of Concrete, Journal of the American Concrete Institute, Vol. 29, No. 5, Nov 1957, pp. 405-414.

[Stark and Bijlaard, 1988] Stark J. W. B., Bijlaard F. S.,: Eurocode 3 - Appendix 6C - Column Bases, TNO Rapport BI-88-094, Delft 1988.

[Stark, 1979] Stark J. W. B.: De berenkening van voetplaten van stalen kolommen, in Dutch, Bouwen met staal, No. 49, Sept. 1979, pp. 21.-28.

[Bouwman at al., 1989] Bouwman, L.P., Gresnigt A.M., Romeijn, A. Onderzoek naar de bevestiging van stalen voetplaten aan funderingen van beton. (Research into the connection of steel base plates to concrete foundations), Stevin Laboratory report 25.6.89.05/c6, Delft 1989.

[Stockwell, 1987] STOCKWELL, F. W. Jr.: Base Plate Design, AISC, National Eng Conf, Proceedings, Paper 49, April / May 1987.

[Stockwell, 1975] Stockwell F. W. Jr.: Preliminary Base Plate Selection, Engineering Journal AISC, Vol. 21, 1975, No. 3, pp. 92-99.

[Wald F at. al., 2000] Wald F., Bouguin V., Sokol Z. Muzeau J. P: Component Method for Base Plate of RHS, Proceedings of the Conference Connections in Steel Structures IV: Steel Connections in the New Millenium, October 22-25, Roanoke 2000.

[Wald, 1999] Wald F.: Column and Anchor Bolts, v Festschrift Commerative Publikation to Ferdinand Tschemmerneg , Institut für Stahlbau, Holzbau und Michbautechnologie, ed. G. Huber, T. Michl, Innsbruck, 1999, p. 317 - 330, ISBN 3-9501069-0-1.

[Wald, 1998] Wald F., Gresnigt A. M., Weynand K., J. P. Jaspart: Application of the Component Method to Column Bases, in Proceedings of  COST C1 Conference Liege 1998, ed. R. Maquoi, p. 155 - 166, Brussels 1999, ISBN 92-828-6337-9.

[Wald, Baniotopoulos, 1998] Wald F., Baniotopoulos Ch. C.: Numerical modelling of column base connection, in COST C1 Conference Liege 1998, p. 497 - 507, Brussels 1999, ISBN 92-828-6337-9.

[Wald, 1995] Wald F.: Patky sloupů, Column Base, CTU, Praha 1995, ISBN 80-01-01337-5, p. 137.

[Wald, 1993] Wald F.: Column-Base Connections, A Comprehensive State of the Art Review, CTU, Praha 1993, pp. 72.